ECU Simple Tuner

Software User Manual

Arduino Uno firmware · Engine Setup Tool · Map Tuner

# 1. Overview

This package is a programmable engine control unit (ECU) for a 4-cylinder engine using wasted-spark ignition and split-batch fuel injection. It has three parts:

- Arduino Uno firmware — runs the engine (spark, injectors, fan, fuel pump, sensors).

- Engine Setup Tool — configure cylinders, crank wheel, sensors, coils, and battery divider; generates ecu_config.h.

- Map Tuner — live ignition/fuel maps, telemetry, calibration, and EEPROM save/load.

Typical workflow: configure hardware once → flash firmware → connect the tuner → read tables from the ECU → tune maps live → save to EEPROM.

# 2. Hardware

## 2.1 Pin map (Arduino Uno)

| Pin | Function | Notes |
| --- | --- | --- |
| D6 | INJ A | Injector bank A (cyl 1+4) |
| D7 | INJ B | Injector bank B (cyl 2+3) |
| D8 | IGN A | Wasted-spark coil A (1+4) |
| D9 | IGN B | Wasted-spark coil B (2+3) |
| D12 | FAN | Cooling fan relay |
| D13 | FP | Fuel pump relay |
| D2 | CRANK | Hall/VR tooth input (INT0) |
| A0 | MAP | Manifold pressure |
| A1 | TPS | Throttle position |
| A2 | ECT / CLT | Coolant temperature (NTC) |
| A3 | IAT | Intake air temperature (NTC) |
| A4 | BAT | Battery via voltage divider |

## 2.2 NTC temperature sensors

Wire each NTC as: 5V → pull-up resistor → ADC pin → NTC → GND.

Default software model: 10 kΩ NTC at 25 °C, beta ≈ 3950, 10 kΩ pull-up. At room temperature ADC should be near 512. Many OEM sensors use a 1 kΩ bias — set pull-up to 1000 Ω in Engine Setup.

## 2.3 Battery voltage divider

Keep the ADC pin under 5 V at maximum pack voltage. Example: 20 kΩ top + 10 kΩ bottom ≈ ratio 3.0. Calibrate in Engine Setup using a multimeter reading and the live BADC value.

# 3. Installing the software

## 3.1 PC requirements

- Python 3.10+ with PySide6 and pyserial

- Arduino IDE (or CLI) for Uno upload

pip install -r requirements.txt

## 3.2 Project files

engine_setup_tool.py     — hardware / sensor configurator

ecu_tuner_aligned.py     — live map tuner

requirements.txt         — Python dependencies

ecu_firmware/

ecu_firmware.ino       — main sketch

ecu_config.h           — generated configuration header

# 4. Engine Setup Tool

Run:

python engine_setup_tool.py

Use the four tabs, then Generate ecu_config.h.

## 4.1 Tab 1 — Engine

- Cylinders (1–8)

- Crank trigger presets: 12-1, 24-1, 24-2, 36-1, 36-2, 60-2, 60-1

- Trigger angle (° BTDC) — filled from the preset; adjust if needed

- RPM limit and fan-on temperature

## 4.2 Tab 2 — Sensors

- CLT / IAT / MAP profile dropdowns (built-in or custom)

- NTC pull-up, R@25°C, and beta for the beta-equation converter

- Sensor Profiler — create custom temp↔ADC or MAP gain tables

## 4.3 Tab 3 — Ignition

- Coil type: Dumb (ECU dwell) or Smart (logic-level pulse)

- Load strategy: Speed-Density (MAP) or Alpha-N (TPS)

## 4.4 Tab 4 — Battery

- ADC Vref (usually 5.00 V) and divider ratio

- Optional: enter known voltage + ADC → Compute ratio

## 4.5 Generate and flash

Click Generate ecu_config.h. Open the ecu_firmware folder in Arduino IDE, select Board → Arduino Uno, and Upload.

# 5. Map Tuner

Run:

python ecu_tuner_aligned.py

## 5.1 Connect

- Select the serial port → Connect (115200 baud).

- On connect the tuner sends GETCFG and GETMAP — config and maps load from the ECU (EEPROM copy in RAM).

- Use Read from ECU anytime to refresh tables.

## 5.2 Live engine panel

- RPM, ignition advance, injector PW, MAP, TPS, ECT, IAT (with ADC), battery

- Crank sync state: idle / searching / locked / lost

- Fan and fuel pump indicators

- Protocol health: RX / ERR counts

## 5.3 Map editing

- Advance Map / Injector Map buttons switch the grid

- Click a cell; + / − small step; Page Up / Page Down large step

- Arrow keys move the selection

- Shift+drag a zone → bilinear interpolation fill

- I re-runs interpolation on the current zone

- Live Tune tracks the live RPM/load cell

Every cell change is sent live to the Arduino as SET:A or SET:I (RAM only until Save EEPROM).

## 5.4 Trigger angle

- Shift+B enters trigger-angle mode

- Left / Right arrows change angle by ±1° and send CFG: live

- Esc or Shift+B again exits the mode

Wheel teeth/missing come from the ECU (GETCFG); only the angle is adjusted in the tuner.

## 5.5 Calibrate Sensors

- Tabs: ECT, TPS (includes closed/full wizard), BAT, IAT, MAP

- Capture Live ADC into the selected row

- Save Calibration to EEPROM pushes tables and SAVE

## 5.6 Bottom bar

| Button | Action |
| --- | --- |
| Read from ECU | GETCFG + GETMAP — fill tuner from ECU RAM/EEPROM |
| Upload Maps | Push both map grids to ECU RAM |
| Save EEPROM | Upload maps + config + cal, then SAVE to EEPROM |
| Export Tune | Save maps to a text file on the PC |

# 6. EEPROM and live data

Live edits (SET:, CFG:) change RAM only. Power loss reverts to last EEPROM contents.

Save EEPROM writes: magic, wheel/config, advance map, injector map, ECT/IAT curves, TPS points.

Boot order on the Uno: apply ecu_config.h profiles → default maps → loadEEPROM() (overrides if magic is valid).

# 7. Serial protocol (summary)

Baud 115200. Telemetry is one comma-separated KEY:value line (~20 Hz).

RPM, MAP, TPS, TMP, IAT, BAT, EADC, TADC, BADC, IADC, MADC,

SYNC, FAN, FP, IGN, PW, LOST, TOOTH, DEG, TERR, DWELL, …

Commands from PC:

SET:A,r,c,val   SET:I,r,c,val   CFG:teeth,missing,trig

UPLOAD:ADV / UPLOAD:INJ + row lines

GETCFG   GETMAP   SAVE

# 8. Troubleshooting

| Symptom | What to check |
| --- | --- |
| Cannot connect | Correct COM port; close Serial Monitor; cable; 115200 |
| No crank sync | Teeth/missing match wheel; sensor air gap; GETCFG values |
| ECT/IAT nonsense | Wiring 5V–pull-up–ADC–NTC–GND; pull-up Ω; ADC near 512 at 25°C |
| Maps reset after power | Use Save EEPROM after tuning |
| BAT reading wrong | Divider ratio in Engine Setup; calibrate with known V + BADC |
| STALE / no telemetry | ECU power; baud rate; TX/RX not swapped |

# 9. Safety notes

- This is experimental software for educational / hobby use.

- Always verify timing with a timing light before relying on the ECU on a running engine.

- Use proper drivers/relays for injectors, coils, fan, and pump — do not drive high current from Uno pins directly.

- Keep the battery sense divider so the ADC pin never exceeds 5 V.

End of manual. For development details, refer to comments in ecu_firmware.ino and the Python sources.