STRIX V2 — TorquEFI Tuner

Software User Manual · UI Reference

STM32F411 Black Pill · USB CDC · 12×22 maps · Flash blob v8

This manual describes the STRIX V2 desktop tuner user interface: connection, live strip, maps, engine settings, setup wizard, flash save, and calibration workflow.

# 1. Overview

STRIX V2 is the PC-side tuner for the TorquEFI / STRIX ECU firmware on STM32F411 (Black Pill). Maps and most settings are pushed live to RAM; Flash writes them to non-volatile memory.

Python 3.10+ with PySide6 and pyserial

USB connection as a virtual COM port (STM32 USB CDC)

12×22 ignition and injection maps with bilinear interpolation

Engine settings stored in .tcal files and (v8) in ECU flash

# 2. Starting the software

From the STRIX_V2 folder:

pip install -r requirements.txt

python main.py

On first run a unique device ID is created under ~/.strix_v2/ for auto-connect.

# 3. Main window layout

## 3.1 Top toolbar

| Control | Action |
| --- | --- |
| Port + Refresh | List serial ports; pick STM32 CDC COM port |
| Connect | Open port, request PROTO + GETMAP, start telemetry |
| Flash | Upload maps to RAM, SAVE to flash, verify with GETMAPSUM |
| ● (link LED) | Green = telem <0.5 s; yellow = stale; red = no data |
| Save log | Export last ~30 s RPM/MAP/TPS/LOAD/SYNC/IGN/PW as CSV |
| Program settings | Optional live-strip fields |
| Sensor calibration | ECT/IAT/MAP/TPS/O2 presets and enables |
| Engine settings | Tabbed hardware / fuel / throttle / I/O / controls |
| Trigger wizard | Suggest teeth/missing while cranking (RPM = 0 required to apply) |
| Setup wizard | Guided 3-page first-time setup |
| Interpolate | Fill selection on the active map |

## 3.2 Live strip

Always shown: RPM, TPS, MAP, ECT, IAT, SYNC. Optional fields (IGN, PW, BAT, AFR, etc.) are chosen under Program settings. Boxes keep a fixed width so the strip does not jump.

SYNC reflects crank lock; CAM status appears when present in telemetry

SYNCQ (firmware) is sync quality 0–100 when the ECU sends it

Crosshairs on both maps follow the live cell when RPM is stable

## 3.3 Map tabs

Ignition (° BTDC) and Injection (ms) are separate tabs. Maps fit the window (no zoom/drag pan). Cell text is bold and centered.

| Action | How |
| --- | --- |
| Edit cell | Click cell, type value, Enter |
| Live to ECU | Each change sends SET:A or SET:I immediately (no Upload button) |
| Undo | Ctrl+Z restores last cell and re-sends to ECU |
| % change | Ctrl+P on selection → percentage adjust |
| Select block | Drag with mouse; Ctrl+click for multi-select where supported |
| Interpolate | Toolbar button fills between corners of selection |

Load axis is MAP (kPa) or TPS (%) from engine settings. MAP values are whole numbers. Intermittent RPM spikes do not move the crosshair.

# 4. Connection

## 4.1 Connect

Select the STM32 Virtual COM Port and click Connect

Auto-connect may pick a known device ID when the port appears

On connect the tuner sends GETPROTO then GETMAP

Maps are loaded into the UI; Flash becomes enabled

## 4.2 Disconnect

Closes the serial port. RAM on the ECU keeps the last values until power-cycle; flash holds the last successful Flash.

# 5. Flash save (verified)

Flash is the only persistent write path:

1. Bulk UPLOAD of ignition and injection tables to RAM

2. SAVE command

3. Wait for OK:SAVE

4. GETMAPSUM compared to UI checksums

If verification fails, a warning is shown. Do not power off until Flash reports OK.

# 6. Engine settings

Open from the toolbar. When RPM > 0, trigger-related fields are locked.

| Tab | Contents |
| --- | --- |
| Core | Wheel profile, cylinders, teeth, missing, trigger angle, firing order, coil type |
| Fuel | Load MAP/TPS/HYBRID, MAP max kPa, fuel pump prime, start inj. prime, injection mode |
| Throttle | Cable vs DBW; idle actuator (disabled / single PWM / dual wire) |
| I/O | ECT/IAT enables, O2 mode, boost mode, VVT mode, fan enable + temperature |
| Controls | RPM limit, hard/soft cut |

Export / Import uses .tcal calibration files. On Apply while connected (and RPM = 0 for core), settings are pushed to the ECU including SET:FPPRIME, SET:INJPRIME, SET:IDLEEN, SET:IDLERPM, CFG, etc.

# 7. Setup wizard

Guided alternative to Engine settings for first configuration:

Page 1 — Trigger: wheel, cylinders, coil, firing order, injection mode, RPM limit/cut

Page 2 — Load / Idle: load axis, throttle type, idle actuator, closed-loop idle enable + target RPM, primes

Page 3 — I/O: ECT/IAT, O2, boost, VVT, fan

Finish & Apply writes .tcal and pushes the ECU. RPM must be 0. DBW shows a safety warning.

# 8. Sensor calibration

Per-sensor presets (Bosch MAP/IAT/CLT, custom ADC endpoints). Enable checkboxes remove sensors from fuel/timing compensation while values can still be viewed. O2: Disabled / Narrowband / Wideband tables.

# 9. Program settings

Choose optional live-strip fields. RPM, TPS, MAP, ECT, IAT, and SYNC are always visible.

# 10. Trigger wizard

Used at RPM = 0. Helps identify crank teeth/missing from observed pattern when cranking is allowed for capture; apply updates CFG on the ECU.

# 11. Closed-loop idle (UI)

Enable under Setup wizard or Engine settings (idle_enable + idle_target_rpm). Pushed as:

SET:IDLEEN,0|1

SET:IDLERPM,850  (500–2000)

Firmware runs PID on DBW throttle adder or idle valve when TPS is low. Telemetry: IDLE, IRPM, ITHR, DASH.

# 12. Keyboard shortcuts

| Key | Action |
| --- | --- |
| Ctrl+Z | Undo last map cell edit |
| Ctrl+P | Percentage change on selection |
| Enter | Commit cell value |

# 13. Typical workflow

1. Wire Black Pill per pinout PDF; flash firmware from CubeIDE

2. Run python main.py → Connect

3. Setup wizard (or Engine settings) at RPM = 0

4. Sensor calibration for MAP/TPS/temps

5. Edit ignition/fuel maps live; watch strip and crosshairs

6. Flash and confirm verified OK before power-down

7. Export .tcal for backup

# 14. Troubleshooting (UI)

| Symptom | What to try |
| --- | --- |
| No COM port | Cable, drivers, device not in DFU; reset Black Pill |
| Link LED red | No telemetry — check CDC, baud not required for USB CDC |
| Maps all zero after read | Flash empty; load defaults and Flash once |
| Flash verify mismatch | Retry Flash; check USB noise; GETMAPSUM vs UI |
| Settings locked | RPM > 0 — stop engine to edit Core/trigger |
| DBW warning | Calibrate pedal/TPS before road use |

# 15. Related files

| File | Description |
| --- | --- |
| STRIX_V2/ | Tuner application |
| STRIX_V2_firmware/ | ecu_app, flash v8, .ioc, linker |
| STRIX_V2_BlackPill_Pinout.pdf | Pin allocations |
| STRIX_V2_complete.zip | Full package |
| ~/.strix_v2/ | Local settings + device ID |

Document version: STRIX V2 UI · Match firmware PROTO:2 and flash blob v8.